ASIC & FPGA Chip Design:

Introduction

## Mahdi Shabany

Department of Electrical Engineering Sharif University of technology

Course Code: 25776

#### Course Outline

- Introduction to IC Design

  - Integrated Circuits (IC) History

  - Digital Design vs. Analog Design

  - ASIC vs. FPGA

  - Design Abstraction and Metrics

  - CMOS as the building block of Digital ASICs

  - Layout

  - Packaging

#### Course Outline

- Introduction to ASIC/FPGA IC Design

  - Integrated Circuits (IC) History

  - Digital Design vs. Analog Design

  - ASIC vs. FPGA

  - Design Abstraction and Metrics

  - CMOS as the building block of Digital ASICs

  - Layout

  - Packaging

###### How to go from Idea/Algorithm to the actual hardware

module RippleCarryAdderII (Cin, X, Y, S, Cout);

parameter n = 4; input Cin; input [n-1:0] X, Y; output [n-1:0] S; wire [n-1:0] C;

Full_Adder stage0  (Cin, X[0], Y[0], S[0], C[1]);

Full_Adder stage1  (C[1], X[1], Y[1], S[1], C[2]);

Full_Adder stage2  (C[2], X[2], Y[2], S[2], C[3]);

Full_Adder stage3 (.Cout(Cout), .Cin(C[3]), .x(X[3]), .y(Y[3]), .S(S[3])); endmodule

ASIC	FPGA

  - Hardware Description Language (HDL) :Verilog

  - Professional Verilog Coding for Synthesis

  - Verification Techniques

  - FPGA Architectures

  - Digital System Design with Xilinx FPGAs

  - ASIC Digital Design Flow (from Verilog to the actual Chip!)

  - Synthesis Algorithms

  - Power Dissipation

  - Power Grid and Clock Design

  - Fixed-point Simulation Methodology

  - Detailed Design Optimization Workshop with ISE (for the fist time!)

Yes

Synthesis

Pre-Layout

Timing	Yes

Alanysis Pass?

Post-Layout

Timing Alanysis Pass?

Yes

NO

NO

Test Bench

NO

Simulation Pass?

RTL Coding

Timing Constraints

Logic verification

APR

Standard Cells

Specifications

Tapeout

Back	 Annotation

- HDL Coding

- Simulation	3. Synthesis

4. Placement & routing

5. Timing Analysis & Verification

Front-End	Back-End

  - In this course we learn all the above steps in detail for

    - ASIC Platform

    - FPGA Platform

- Hardware Description Language

##### Verilog Fundamentals

  - Language Fundamentals

  - Modeling Combinational/Sequential Logic Circuits

  - Modeling Finite State Machines

##### Verilog for Verification

  - Verification/Simulation techniques with test‐benches

##### Verilog for synthesis

  - Verilog Styles for Synthesis

  - Architectural techniques for high‐speed designs

    - Parallel proc., pipelining, retiming, …

  - Implementations of common operations

    - Complex multiplication, division, complex norm, CORDIC

  - Fundamentals of fixed‐point realization

### PLDs & FPGA Architectures

FPGA Technologies

  - SPLDs (PAL and PLA architectures)

  - Commercial CPLD Architectures

  - SRAM/LUT Based FPGAs

  - Anti‐fuse/MUX Based FPGAs

  - Flash Based FPGAs

FPGA Architectures

  - Heterogeneous/Homogeneous FPGAs

  - Fine‐grained, coarse‐grained and platform FPGAs

FPGA Elements & Design Trade‐offs

  - Logic Cells Common Architectures

  - Programmable Routing Channels Design

  - I/O & Pad architectures

Commercial FPGAs

  - Altera (FLEX 10K, Stratix III) , Xilinx (XC4000, Virtex II,4,5), Actel (Act3, Axcelerator)

### Advanced Digital System Design with Xilinx FPGAs

- Design Creation

- Synthesize

- Simulation

- Constraints Entry

- Implementation

- Implementation Results Analysis – Timing Analysis

- Implementation Results Analysis – Power Analysis

- Implementation Results Improvement

- Device Configuration and Programming

- Design Debugging

- Core Generator

- CORE Generator Tool

- Intellectual Property (IP) Cores

- CORE Generator Tool files

- Design Flows

- Defining Memory Contents for RAM and ROM

- Defining Coefficient Values in a COE File

- ASIC Design Flow:

HDL Coding & Verification

Synthesis & Timing Optimization

  - Complete Synopsys Design Complier Design Flow

Physical Design

  - Cadence First Encounter

  - Floorplan (Initial floorplan and power planning)

  - Placement (Full‐scale floorplan and clock tree insertion)

  - Routing (power routing & Nanoroute)

  - Timing Closure (Analysis & Optimization of setup and hold time violations)

  - Fill (Filler Cells, Metal Fill, and Verify Geometry)

### CAD Tool Algorithms:

##### Synthesis Algorithms

Two-level Optimization

Multi-level Logic Optimization

Technology Mapping

### Power Dissipation

###### Power Dissipation concept

- Dynamic Power

- Static Power

- Challenges

- Power Grid and Clock Design

- Power Distribution Design

  - Introduction

  - IR Drop

  - Ldi/dt Drop

- Decoupling Capacitances

- Clock Considerations

- PLL/DLL Architecture

##### Prerequisites:

###### Only Digital Logic!

All the skills you need will be taught in the course

- Softwares you will learn:

Altera Quartus

Xilinx ISE

Mentor Graphics Modelsim

TCL Scripting

Synopsys Design Compiler

Cadence SOC Encounter

##### Implementation Platform:

Altera DE2 Board

Atlys Xilinx Board

- You will do several practical assignments including Verilog Coding, FPGA implementation and testing based on the DE2 and Atlys Boards

##### Assignments & Labs (12)

###### 4 implementation assignments on DE2 Board (8 Marks)

6 Implementation assignments on Atlys boards (12 Marks)

2 Paper Assignments (4 marks)

- 1 comprehensive project from A to Z (5 marks)

Soft deadlines: 20% daily penalty for the late submissions

| Section | Quantity | Value each | Value |
| --- | --- | --- | --- |
| Assignments | 12 | 2 | 24 |
| Project | 1 | 6 | 6 |
| Midterm | 1 | 30 | 30 |
| Final Exam | 1 | 40 | 40 |

100

Total:

If both the midterm and final marks are less than 50%, the total mark is calculated only based on the exams and assignments will not be taken into account.

  - HDL:

Thomas & Moorby, The Verilog Hardware Description Language, 3rd ed, Kluwer Academic.

Introduction to Logic Synthesis using Verilog HDL, Robert Reese, Mitchell Thornton, 2006.

Advanced FPGA Design, Architecture, Implementation, and Optimization, Steve Kilts, 2007.

  - IC Design Flow:

Course Lecture notes, 2014

Digital IC Design Flow, provided by the instructor, 2014

# Course Outline

#### Course Outline

- Introduction to ASIC/FPGA IC Design

  - Integrated Circuits (IC) History

  - Digital Design vs. Analog Design

  - ASIC vs. FPGA

  - Design Abstraction and Metrics

  - CMOS as the building block of Digital ASICs

  - Layout

  - Packaging

Discrete Circuits

Limited applications

Power hungry

Large area

Moderate speed

Integrated Circuits

Many applications

Low power

Small area

High speed

1940	1945	1947	1951	1955	1957	1958	1959	1961	1962	1968	1970	1971

Ohl built the PN Junction: 1940 Shockley Lab was established: 1945

A picture of the filed patent

Brattain and Bardeen invented the first transistor (US Patent 2524035): 1947

1940	1945	1947	1951	1955	1957	1958	1959	1961	1962	1968	1970	1971

- 1951: Shockley invented the first junction transistor for mass production

A picture of the filed patent

(US Patent 2623105)

1940	1945	1947	1951	1955	1957	1958	1959	1961	1962	1968	1970	1971

###### پیوست

Shockley

###### به آزمایشگاه

Noyce

###### :1955

Hoerni

###### با کمک

Fairchild

###### را به هدف تاسیس

Shockley

###### آزمایشگاه

Noyce

###### :1957

ترک کرد

Gorden Moore و

###### ابداع کرد و به این

تکنیکی را برای افشاندن ناخالصی ها بر روی Si

Hoerni

###### :1958

بسازد

SiO2

###### ترتیب توانست یک ترانزیستور مسطح با کمک عایق

اولین مدار مجتمع را با استفاده از ترانزیستورهای مسطح ساخت

Noyce

###### :1959

1940	1945	1947	1951	1955	1957	1958	1959	1961	1962	1968	1970	1971

###### کار می کرد ایده مدار مجتمع یک

Texas Instrument (TI)

###### که در

Jack Kilby

###### :1959

پارچه را مطرح کرد. در این طرح قطعات به وسیله سیم های لحیم شده به یکدیگر متصل

(U.S Patent (

###### به عنوان مقاومت استفاده می شد

می شدند و از دیودهای PN

)3,138,473

A picture of the filed patent

)50$

###### (هر کدام

منطقی را تولید انبوه کردند

اولین IC

Fairchild و

TI :1961

###### شانزده بیتی ساخته شد

Ripple Carry Adder

###### :1962

Fairchild bipolar RTL Flip-Flop	RCA 16-transistor MOSFET IC

:1968 Noyce و Moore موسسه Fairchild را رها کرده و Intel را پایه گذاری کردند

کرد تولید را Static RAMs 256-bit , Fairchild :1970

را فروخت

Dynamic RAMs 1-K bit , Intel :1970

Fairchild 4100 256-bit SRAM	Intel 1103 1K-bit DRAM

1940	1945	1947	1951	1955	1957	1958	1959	1961	1962	1968	1970	1971

###### را تولید کرد

4004 ,

Intel

###### :1971

اولین کامپیوتر قابل برنامه ریزی چند منظوره برای شرکت ماشین حساب ژاپنی

1971	………………………………………………………………………………………	2010

1 Billion Transistors

Projected

Pentium® III Pentium® II

i486

Pentium® Pro

Pentium®

i386 80286

8086

K

1,000,000

100,000

10,000

1,000

100

10

1

Source: Intel

Number of Transistors doubled every 18 months!

(Moore’s Law)

Scaling still continues…

1975	1980	1985	1990	1995	2000	2005	2010

1971	………………………………………………..………………………	2015

###### Amazingly visionary – million transistors/chip barrier was crossed in the 1980’s.

  - 2300 transistors, 1 MHz clock (Intel 4004) - 1971

  - 16 Million transistors (Ultra Sparc III)

10 gates

1000 gates

10,000 gates

> 10k gates

SSI:

MSI: LSI: VLSI:

Integration Levels

42 Million, 2 GHz clock (Intel P4) - 2001

  - 140 Million transistor (HP PA-8500)

Early Designs	Advanced Designs

(1971)	(2017)

- 1000 Transistors (1 MHz)

- Process: 10um

- Fully Handcrafted

- Manual Layout

- Individually Optimized

- Intel Corei7 -7700K(~ 4.2 GHz)

- Process: 14nm

- Fully Automated

- Automated Layout

- Hierarchical Design

Digital Very Large Scale Integration (VLSI) (Not Analog!)

up frequency doubles every 2 years

Power density too high to keep

junctions at low temp

| Year | 1999 | 2002 | 2005 | 2008 | 2011 | 2014 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Feature size (nm) | 180 | 130 | 100 | 70 | 50 | 35 | 14 |
| M trans/cm2 | 7 | 14-26 | 47 | 115 | 284 | 701 | 900 |
| Chip size (mm2) | 170 | 170-214 | 235 | 269 | 308 | 354 | 460 |
| Signal pins/chip | 768 | 1024 | 1024 | 1280 | 1408 | 1472 | ~1600 |
| Clock rate (MHz) | 600 | 800 | 1100 | 1400 | 1800 | 2200 | 4200 |
| Wiring levels | 6-7 | 7-8 | 8-9 | 9 | 9-10 | 10 | 10 |
| Power supply (V) | 1.8 | 1.5 | 1.2 | 0.9 | 0.6 | 0.6 | 0.8 |

- Technology shrinks by ~0.7 per generation

- With every generation can integrate 2x more functions on a chip; chip cost

does not increase significantly

- Cost of a function decreases by 2x

- But …

  - How to design chips with more and more functions?

  - Design engineering population does not double every two years…

- Hence, a need for more efficient design methods

  - Exploit different levels of abstraction

Silicon Cylinder

Silicon Wafer

Patterned Silicon Wafer

Circuit Core

Unpackaged Die

Packaged Die	Final Test

Die

Package

Circuit core

Packaged Die

Cavity

Bond wire

Trace

Patterned Silicon

Wafer

PAD

Lead

Circuit Core

- Course Outline

- Introduction to ASIC/FPGA IC Design

  - Integrated Circuits (IC) History

  - Digital Design vs. Analog Design

  - ASIC vs. FPGA

  - Design Abstraction and Metrics

  - CMOS as the building block of Digital ASICs

  - Layout

  - Packaging

Analog/RF

Voice

Power

Amplifier

Modulator

Transmitter:

- Receiver:

High-frequency carrier

Low-Noise

Amplifier

Audio

Amplifier

Demodulator

Down Converter

Analog/RF

### Transmitter:

Digital (Baseband)	Analog/RF Front-end

Modulator

Pulse shaping

Coding Interleaving

Voice Compression

ADC

Voice

- Receiver:	This Course

Power Amplifier

Down Converter

ADC

Demodulator

Equalizer

Low-Noise

Amplifier

DAC

Voice Decompression

De-interleaving Decoding

Audio

Amplifier

Digital (Baseband)

##### Digital Design Advantages:

###### More noise reliability

Allows signal decoding and amplification

Allows coding to achieve higher performance

Allows encryption for higher security

Provides a perfect vehicle for digital signal processing

Allows modular chip design

Enjoys the benefit of advanced sophisticated CAD tools

Can be verified on programmable devices before tape-out

Provides a platform to merge multiple networks such as telephone, terrestrial TV and computer networks.

- Specific application (e.g., Digital Image Processor)

As opposed to a general microprocessor

- Design requirements

Images to be processed per second

Required processing

Interfacing

Dimension, power, price,…

- General design and simulation

- RTL level design to verify and simulate the design

- Higher levels: Few complicated blocks

- Lower levels: Many simple blocks

To avoid any problem in lower levels, many simulations in higher levels

RTL

Idea

IC Fabrication

Wafer (hundreds of dies)

Architecture Design

Sawing & Packaging

Block diagram

Final chips

Testing

Good chips

Bad chips

customers

Circuit & Layout Design

Layout

  - Printed Circuit Board (PCB)

  - Embedded Software

    - Microprocessor (general Purpose)

    - Microcontroller

    - Digital Signal Processor (DSP)

  - Programmable Logic Devices

    - Simple Programmable Logic Device (SPLD)

    - Complex Programmable Logic Device (CPLD)

    - Field Programmable Gate Arrays (FPGAs)

  - Application Specific Integrated Circuits (ASICs)

##### DSPs

- Easy to program (usually standard C)

- Very efficient for complex sequential math-intensive tasks

- Fixed datapath-width. Ex: 24-bit adder, is not efficient for 5-bit addition

- Limited resources

Microprocessors/Microcontrollers

- Lessens the risk of system development by reducing design complexity

- Fixed hardware so more effort on developing a good code

- Fixed HW not suitable for high level of parallelism/computations

- Many sequential nature

- One order of magnitude less performance than to-date FPGA/ASICs

Gate Array

Standard cell

PAL

PLA

Full-Custom

Semi-Custom

FPGA

CPLD

SPLD

ASIC

PLDs

Digital IC

This course

The white portions of the timeline bars indicate that although early incarnations of these technologies may have been available, they weren’t enthusiastically received by the engineers working in the

trenches during this period. For example, although Xilinx introduced the world’s first FPGA as early as 1984, design engineers didn’t really start using it until the early 1990s.

# Course Outline

#### Course Outline

- Introduction to ASIC/FPGA IC Design

  - Integrated Circuits (IC) History

  - Digital Design vs. Analog Design

  - ASIC vs. FPGA

  - Design Abstraction and Metrics

  - CMOS as the building block of Digital ASICs

  - Layout

  - Packaging

- Field Programmable Gate Array (FPGA) Advantages:

Fast programming and testing time by the end user (instant turn-around)

Excellent for prototyping

  - Easy to migrate from prototype to the final design

Can be re-used for other designs

Cheaper (in small volumes)	lower start-up costs

Re-programmable

Lower financial risk

Ease of design changes/modifications

Cheaper design tools

- FPGA Drawbacks:

Slower than ASIC (2-3 times slower)

Power hungry (up to 10 times more dynamic power)

Use more transistors per logic function

More area (20 to 35 times more area than a standard cell ASIC)

- Application Specific Integrated Circuit (ASIC) Advantages:

Faster

Lower power

Cheaper (if manufactured in large volumes)

Use less transistors per logic function

- ASIC Drawbacks:

Implements a particular design (not programmable)

Takes several months to fabricate (long turn-around)

More expensive design tools

Very expensive engineering/mask cost for the first successful design

- The ASIC is accompanied by increasing nonrecurrent engineering (NRE) costs which meant

that there was an increased emphasis on “right first time” design.

- These NRE costs is largely due to the cost of generating masks as it is becoming more expensive to generate the masks for finer geometries needed by shrinking silicon technology dimensions.

- Expensive & time consuming

fabrication in semiconductor foundry

- Designed all the way from

behavioral description to

physical layout

ASIC

Application Specific Integrated Circuit

- Bought off the shelf & reconfigured

by the end designers

- No physical layout design

- Design ends with a bitstream used

to configure a device

FPGA

Field Programmable Gate Array

ASICs	FPGAs

Off-the-shelf

High performance

Low development cost

Low power

Short time to market

Low cost in

high volumes

Re-configurability

# Course Outline

#### Course Outline

- Introduction to ASIC/FPGA IC Design

  - Integrated Circuits (IC) History

  - Digital Design vs. Analog Design

  - ASIC vs. FPGA

  - Design Abstraction and Metrics

  - CMOS as the building block of Digital ASICs

  - Layout

  - Packaging

  - Divide-and-Conquer

SYSTEM

MODULE

+

GATE

CIRCUIT

S

n+

DEVICE

G

D

n+

Design modules once

Instantiate them thereafter

Standard Cells

- Already laid out

Avoid re-design

Same as programming

- Designer cares about module’s:

Functionality

Delay characteristics

Area

- NOT:

How the module was designed

Detailed solid-state behavior

This Course

VLSI Course

Circuit Logic

Register Transfer Algorithmic

Architectural (System)

Y-Chart

System Level

Algorithmic Level

I/O

Processor

RAM

ROM

A:=A*B+F IF(C=TRUE) THEN A:=A+2*F

ELSE A:=A-1 ENDIF

MUX

ROM

A

L

U

C

Q

D	Qn

|

&

Register

Register Transfer Level	Logic Level

Circuit Level

##### Performance Metrics of a Digital Chip:

  - Cost

Single Die

Wafer

Diameter: 10-30 cm

Thickness: 1 mm

NRE (fixed) costs - design effort

RE (variable) costs - cost of parts, assembly, test

  - Speed

Delay (ns) → Operating Frequency (MHz)

  - Power Dissipation

  - Energy to Perform a Function

Energy per bit (nJ/b)

  - Reliability

Noise immunity

Noise margin

  - Scalability

Larger Designs

  - Time-to-Market

##### Performance Metrics of a Digital Chip:

  - Cost

Single Die

Wafer

NRE (fixed) costs - design effort

RE (variable) costs - cost of parts, assembly, test

  - Speed

Delay (ns) → Operating Frequency (MHz)

  - Power Dissipation

  - Energy to Perform a Function

Energy per bit (nJ/b)

  - Reliability

Noise immunity

Noise margin

  - Scalability

Larger Designs

  - Time-to-Market

- NRE (non-recurring engineering) costs

  - Fixed cost to produce the design

    - Design effort

    - Design verification effort

    - Mask generation

  - Influenced by the design complexity and designer productivity

  - More pronounced for small volume products

- RE (Recurring costs) – proportional to the product volume (i.e., Variable)

  - Silicon processing

    - also proportional to chip area

  - Assembly (packaging)

fixed cost

Cost per IC =  variable cost per IC +	-----------------

volume

Test

cost of die + cost of die test + cost of packaging variable cost =	----------------------------------------------------------------

final test yield

cost of die

cost of wafer

=	-----------------------------------

dies per wafer × die yield

 × (wafer diameter/2)2	 × wafer diameter Dies per wafer = ---------------------------------- 		---------------------------

die area	 2 × die area

α depends on the complexity of the manufacturing process (roughly proportional to the number of masks)

die yield = (1 + (defects per unit area × die area)/)-

###### Example

  - Wafer size of 12 inches, die size of 2.5 cm2, 1 defects/cm2,

    -  = 3 (measure of manufacturing process complexity)

  - 252 dies/wafer (remember, wafers round & dies square)

  - Die yield of 16%

  - 252 x 16% = only 40 dies/wafer die yield !

###### Die cost is a strong function of the die area

  - Proportional to the third or fourth power of the die area

Cost of die = f(die area)4

| Chip | Metal layers | Line width | Wafer cost | Defects /cm2 | Area (mm2) | Dies /wafer | Yield | Die cost |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 386DX | 2 | 0.90 | $900 | 1.0 | 43 | 360 | 71% | $4 |
| 486DX2 | 3 | 0.80 | $1200 | 1.0 | 81 | 181 | 54% | $12 |
| PowerPC 601 | 4 | 0.80 | $1700 | 1.3 | 121 | 115 | 28% | $53 |
| HP PA 7100 | 3 | 0.80 | $1300 | 1.0 | 196 | 66 | 27% | $73 |
| DEC Alpha | 3 | 0.70 | $1500 | 1.2 | 234 | 53 | 19% | $149 |
| Super SPARC | 3 | 0.70 | $1700 | 1.6 | 256 | 48 | 13% | $272 |
| Pentium | 3 | 0.80 | $1500 | 1.5 | 296 | 40 | 9% | $417 |

##### Performance Metrics of a Digital Chip:

- Cost

Single Die

Wafer

NRE (fixed) costs - design effort

RE (variable) costs - cost of parts, assembly, test

- Speed

Delay (ns) → Operating Frequency (MHz)

- Power Dissipation

- Energy to Perform a Function

Energy per bit (nJ/b)

- Reliability

Noise immunity

Noise margin

- Scalability

Larger Designs

- Time-to-Market

###### Frequency of operation: 1/T

  - Dependent on the propagation delay of signal through the logic

  - Time to get the data out/in of the registers

  - Clock uncertainty

###### tp is a typical measure (not an accurate one)

  - the delay experienced by a signal when passing through a gate

  - 50% transition points of the input and output waveforms

    - Two types LH and HL

    - Good for comparison

###### Rise time/fall time affects delay

  - 10% - 90% definitions

Vin	Vout

Vin

m

50%

Propagation delay

tp = (tpHL + tpLH)/2

tpHL

tpLH

t

Vout

90%

t

rm

50%

signal slopes

10%

t

f

t

t

r

input wavefor

outpu

wavefo

© M. Shabany, ASIC & FPGA Chip Design

##### Performance Metrics of a Digital Chip:

- Cost

Single Die

Wafer

NRE (fixed) costs - design effort

RE (variable) costs - cost of parts, assembly, test

- Speed

Delay (ns) → Operating Frequency (MHz)

- Power Dissipation

- Energy to Perform a Function

Energy per bit (nJ/b)

- Reliability

Noise immunity

Noise margin

- Scalability

Larger Designs

- Time-to-Market

Ppeak = ipeakVsupply = max[p(t)]

Peak transient power

  - Power line sizing, decoupling, etc.

##### Average power

  - Battery current delivery, cooling system

##### Static power vs. Dynamic power

  - Static current  no computation, etc

  - Dynamic current  Switching on/off the gates

- The higher the number of switching events, the higher the dynamic power consumption

- Dynamic Energy  Amount of energy that is needed to be spent to do a job

  - Time is no matter

###### Propagation delay and the power consumption of a gate are related

- Propagation delay is (mostly) determined by the speed at which a given amount of energy can be stored on the gate capacitors

  - the faster the energy transfer (higher power dissipation) the faster the gate

###### For a given technology and gate topology, the product of the power

consumption and the propagation delay is a constant

  - Power-delay product (PDP) – energy consumed by the gate per switching event

###### An ideal gate is the one that is fast and consumes little energy, so the ultimate quality metric is

  - Energy-delay product (EDP) = power-delay

##### Performance Metrics of a Digital Chip:

- Cost

Single Die

Wafer

NRE (fixed) costs - design effort

RE (variable) costs - cost of parts, assembly, test

- Speed

Delay (ns) → Operating Frequency (MHz)

- Power Dissipation

- Energy to Perform a Function

Energy per bit (nJ/b)

- Reliability

Noise immunity

Noise margin

- Scalability

Larger Designs

- Time-to-Market

- Noise : unwanted variations of voltages and currents at the logic nodes

###### from two wires placed side by side

    - Capacitive coupling

      - Voltage change on one wire can

influence signal on the neighboring wire

      - Cross talk

    - Inductive coupling

      - Current change on one wire can influence signal on the neighboring wire

###### from noise on the power and ground supply rails

    - May influence signal levels in the gate

v(t)

i(t)

VDD

Noise Margins

- For robust circuits, want the “0” and “1” intervals to be as large as possible

NMH = VOH - VIH

VDD	VDD

VOH

Noise Margin High

Noise Margin Low

"1"

VIH

Undefined

Region

NML = VIL - VOL

VIL

VOL

Gnd

"0"

Gnd

Noise margin represents the levels of noise that can be sustained

when gates are cascaded

Gate Output	Gate Input

## Noise Immunity

  - Noise margin expresses the ability of a circuit to overpower a noise source

    - Noise sources:	supply noise, cross talk, interference, offset

  - Absolute noise margin values are deceptive

    - Floating node is more easily disturbed than a node driven by a low

impedance (in terms of voltage)

  - Noise immunity expresses the ability of the system to process and transmit information correctly in the presence of noise (noise rejection)

  - For good noise immunity, the signal swing (i.e., the difference between VOH and VOL) and the noise margin have to be large enough to overpower the impact of fixed sources of noise

# Course Outline

#### Course Outline

- Introduction to ASIC/FPGA IC Design

  - Integrated Circuits (IC) History

  - Digital Design vs. Analog Design

  - ASIC vs. FPGA

  - Design Abstraction and Metrics

  - CMOS as the building block of Digital ASICs

  - Layout

  - Packaging

##### Bipolar Junction Transistor (BJT)

  - Small current in Base drives a larger current b/w Emitter & Collector.

Mostly Analog

###### 

Mostly Digital

Quiescent power dissipation due to the Base current

High power dissipation limits the number of transistors on a single chip

Not suitable for Very Large Scale Integration (VLSI)

##### Metal Oxide Semiconductor Field Effect Transistors (MOSFET)

- Come with almost zero control current (Gate voltage controls the drain current)

Higher integration

Much lower power consumption than BJT

Come in two flavors : n-MOS (n-type dopants), p-MOS (p-type dopants)

- Complementary Metal Oxide Semiconductor (CMOS)

Utilizing both n-MOS and p-MOS transistors

  - Transistors are built on a silicon substrate

  - Silicon is a Group IV material → covalent bonds with 4 adjacent atoms

  - Silicon is a poor conductor → can be raised by adding dopants

Si	Si	Si

- Group V dopants

- Five valence electrons

- Extra electron free to move

- Negative carrier

- Example:

Arsenic, Phosphorus

n-type

Si

Si

Si	Si

Si	Si

  - ype

- Group III dopants

- 3 valence electrons

- Missing electron(hole) free to move

- Positive carrier

- Example:

Boron

Si	Si -	Si

Si	Si	Si

+

Si	As+	Si	Si	B - 	Si

Si	Si	Si	Si	Si	Si

  - Four terminals:

    - Gate

    - Source

    - Drain

    - Body

Body

Source

Gate

Drain

Polysilicon SiO 2

n+

G

  - Consists of :

    - Gate (Metal (old), Polysilicon (now))

    - Insulating layer (SiO2 (oxide- glass))

    - Source (n+ in nMOS, p+ in pMOS)

    - Drain (n+ in nMOS, p+ in pMOS)

    - Body (conductor)

    - n+: Heavily doped n-type

    - P+: Heavily doped p-type

D	S

bulk Si

p

n+

B

nMOS

  - Gate–oxide–body stack looks like a capacitor

  - Body is commonly tied to ground (0 V)

  - Gate at low voltage:

    - Body is at low voltage

    - Source-body diode is OFF

    - Drain-body diode is OFF

    - No current flows

    - Transistor is OFF

Body

Source

Gate

Drain

bulk Si

p

n+

n+

OFF

- Gate at high voltage:

  - Positive charge on gate

  - Negative charge attracted to body

  - Inverts a channel under gate to n-type

  - Current flow in this channel b/w source and drain when drain voltage is nonzero

  - Transistor is ON

Body

Source

Gate

Drain

bulk Si

p

n+

n+

ON

- Similar to nMOS with reversed doping and voltages

- Body is commonly tied to high voltage (VDD)

- Gate low: transistor ON

- Gate high: transistor OFF

- Bubble indicates inverted behavior

Body

Source

Gate

Drain

Polysilicon SiO 2

p+

p+

G

bulk Si

p

D	S

B

pMOS

- L is the channel length

- L : Process parameter, technology

- Smaller L → Faster transistors → higher speed circuits

- Typical process values: 0.35μm, 0.18μm, 0.13μm, 90nm, 60nm, …

- VDD decreases by technology

  - 1.5 V for 0.18 μm

  - 1.2 V for 0.13 μm

- Lower VDD saves power consumption

- GND = 0 V

  - Silicon wafer is the base material

    - Diameter: 10-30 cm

    - Thickness: 1 mm

  - CMOS : both nMOS and pMOS transistors fabricated on a single wafer

  - Wells : special regions to separate bulks of nMOS and pMOS

SiO2

p substrate

n well

p+

p+

n+

n+

n+ diffusion p+ diffusion polysilicon metal1

nMOS transistor	pMOS transistor

  - Substrate must be tied to GND and n-well to VDD

  - Poor connection of metal to lightly-doped semiconductor (Shottky Diode)

GND

V DD

well tap

p substrate

substrate tap

n well

n+

p+

p+

n+

n+

p+

Use heavily doped well and substrate contacts

Cross Section along Dashed Line

Top View

VDD

GND

A

•ارزان

•اتصال  subبه زمین مشترک

NWELL

PWELL

•دو درجه آزادی

•ایزوله از sub

Twin-WELL

ارزان

اتصال subبه بزرگترین منبع

گران

# Course Outline

#### Course Outline

- Introduction to ASIC/FPGA IC Design

  - Integrated Circuits (IC) History

  - Digital Design vs. Analog Design

  - ASIC vs. FPGA

  - Design Abstraction and Metrics

  - CMOS as the building block of Digital ASICs

  - Layout

  - Packaging

  - Chips are specified with set of masks

  - Minimum dimensions of masks determine transistor size (and hence

speed, cost, and power)

  - “Feature size”

Lmin= distance between source and drain (channel)

(minimum width of Polysilicon)

  - Feature size improves 30% every 3 years or so

SiO2 gate oxide

polysilicon

gate

W

tox

n+

L

n+

p-type body

Feature Size

  - Can integrate 2× more functions per chip →	2× less cost per function

  - Normalize for feature size when describing design rules (

 Lmin

/ 2 )

    - E.g., 	= 90nm in 0.18 μm process

  - Each layout consists of various levels described by different colors.

Top View

P substrate

wafer

n well

Cross Section View

  - Circuit engineer designs a circuit

  - Process engineer fabricates the design

Deign Rule Check (DRC)

is the interface between them

  - DRC : constraints on patterns in terms of minimum width and separation

  - DRC: guarantees that the circuit to be manufacturable

  - Designed and laid-out standard cells

  - They are placed & abutted in a chip

Metal 2

Via 12

Metal 1

Several metal layers used for routing

  - Standard cells are automatically placed and routed using different metal

layers through the corresponding CAD tools

  - Start with the pin count (# of I/O and VDD, VSS pads)

  - Pads are already designed (provided in the design kit)

I/O Pads

I/O Pads

VDD Core

Corner Pads

I/O Pads

Corner Pads

VSS Core

Core

Design

I/O Pads

Corner Pads

Corner Pads

Layout View	Design View

# Course Outline

#### Course Outline

- Introduction to ASIC/FPGA IC Design

  - Integrated Circuits (IC) History

  - Digital Design vs. Analog Design

  - ASIC vs. FPGA

  - Design Abstraction and Metrics

  - CMOS as the building block of Digital ASICs

  - Layout

  - Packaging

##### Package:

    - Die interface to outside world

- Removes heat from chip

- Mechanical support

- Protects die against humidity

×	Introduces delay/parasitics to the chip

##### Advanced Package Requirements:

    - Electrical: Low Parasitics

    - Mechanical: Reliable and Robust

    - Thermal: Efficient Heat Removal

    - Economical: Cheap

Pad Pitch

Substrate

Die

Pad

Lead Frame

Die

Solder bumps

Interconnect Layers

Substrate

Wire Bonding

- Small pin count

- Must be attached serially

- Large inductance

- Cross talk b/w wires

- Pads on four sides of the chip

Flip-Chip Bonding

- Large pin count

- Connections made simultaneously

- Superior electrical performance

- Small cross talk

- Pads at any position on the chip

##### Package-Socket-PCB interconnection:

- Performance bottlenecks:

###### Chip itself

  - Package

  - Socket

  - Board

Package

Socket

Through-Hole Mounting

- Mechanically reliable

×	Low package density

×	Limits routing on the board

- Dualin-Line (DIP) (up to 64 pins)

- PGA (up to 400 pins)

Surface Mount

- More wiring space

- Higher package density

- Chips at both sides of the board

×	Weak chip-board connection

×	Non-accessible pins for testing

###### For very large pin-counts, even surface-mount packaging is not enough!

  - Ball Grid Array (BGA)

Solder bumps are used to connect both the die to the package substrate and the package to the board

##### Package Material:

  - Plastics (cheaper)

  - Ceramic (Better heat removal)

##### Package Types:

Pin Grid Array (PGA)

Quad Flat Pack (QFP)

Die

Solder bumps

Interconnect Layers

Substrate

Ball Grid Array (BGA)	Plastic Leader Chip Carrier

(PLCC)